| Issued: | ENGINEERING DIVISION (BỘ PHẬN KỸ THUẬT) | Index Nº: |
| --- | --- | --- |
| January 2023 | POLICIES & PROCEDURES (CHÍNH SÁCH VÀ QUY TRÌNH) | SSC017 |
| Revised: |  | Approved By: |
|  |  | GM.DOE |
|  | SAFETY & SECURITY (AN TOÀN VÀ AN NINH) |  |
| New Revision Due: |  |  |

2 pages

Subject: Safety Harness 
(Chủ đề: Dây đai an toàn)

# POLICY

To ensure safe workplace procedures are followed when checking and wearing a Safety Harness.

Đảm bảo các quy trình làm việc an toàn được tuân theo khi kiểm tra và đeo dây an toàn.

WARNING: YOUR LIFE IS DEPENDENT UPON THESE PROCEDURES

PHỤ LỤC CẬP NHẬT THỦ TỤC THỦ TỤC NÀY PROCEDUR

- Follow all steps listed in the Safety Harness Checklist following.

Thực hiện theo tất cả các bước được liệt kê trong Danh sách kiểm tra an toàn sau đây.

- Circle Yes to each procedure and Sign on completion.

Vòng tròn Có cho mỗi thủ tục và Đăng ký hoàn thành.

- Check list must be completed in full and signed prior to the commencement of any task requiring the wearing of a safety harness.

Danh sách kiểm tra phải được điền đầy đủ và ký trước khi bắt đầu bất kỳ nhiệm vụ nào đòi hỏi phải đeo dây an toàn.

# REFERENCE DOCUMENTS

POLICY AND PROCEDURE

SUPPORT DOCUMENTS

SAFETY HARNESS CHECK LIST – TO BE COMPLETED AND SIGNED PRIOR TO WORK.

| - Read all labeling: Đọc tất cả nhãn dán | YES | NO |
| --- | --- | --- |
| - Check fall indicator: Kiểm tra | YES | NO |
| - Check webbing for abrasions or damage: | YES | NO |
| - Check buckles for damage or distortion: Kiểm tra khóa xem có bị hư hỏng hoặc biến dạng. | YES | NO |
| 5. Check D rings for damage or distortion. | YES | NO |
| 6. Check for discolouration on all webbing. | YES | NO |
| 7. Check all stitching for signs of stress and fractures. | YES | NO |
| 8. Hold harness out in front of you before putting it on. | YES | NO |
| 9. Place right arm through right loop. | YES | NO |
| 10. Place left arm through left loop. | YES | NO |
| 11. Adjust vertical shoulder straps. | YES | NO |
| 12. Pull outer web through buckle to adjust evenly on both sides. | YES | NO |
| 13. Connect chest strap buckle. | YES | NO |
| 14. Ensure rescue loops are at central chest level and behind chest. | YES | NO |
| 15. Check location of dorsel “D” i.e. high and central. | YES | NO |
| 16. Locate rear strap. | YES | NO |
| 17. Pull leg straps through and connect to side buckles. | YES | NO |
| 18. Adjust leg straps by adjusting outer strap. | YES | NO |
| 19. Place hands through leg straps to ensure straps are firm. | YES | NO |
| 20. Fold excess strap into elastic. | YES | NO |
| 21. Tighten chest strap. | YES | NO |
| 22. Locate extension strap on left shoulder. | YES | NO |
| 23. Your harness is now fitted correctly - pull away extension strap from Velcro. | YES | NO |
| 24. Connect fall arrest device to extension strap before entering hazard area. | YES | NO |

.

Signed ……………………….………………….. Date …………………………

Position …………………………………………………………………………..