| Issued: | KITCHEN | Index Nº: |
| --- | --- | --- |
| March 2018 | POLICIES & PROCEDURES | KT-035 |
| Revised: |  | Approved By: |
| New Revision Due: |  |  |

1 page

Subject: Vacuum Packaging of Foods

Objective

To make sure the correct usage of vacuum Packed Foods

Procedures

Vacuum packaging of foods

Although vacuum packing of cooked meat can protect meat from contamination and increase the shelf life of the product, under certain circumstances a bacterium called Clostridium Botulinum may grow. This bacterium can produce a very powerful poison (toxin) in the food which causes Botulism – a frequently fatal form of food poisoning. As this bacterium prefers to grow without air, vacuum packed meats are more at risk. The bacterium is also able to grow and produce toxin at low temperatures and therefore it is important that these products are kept under controlled refrigeration

Vacuum Packaging of Foods Procedure

Factors for controlling the hazards posed by vacuum packing cooked meat are as follows

- The vacuum packing machine should ideally be used only for cooked meats. If raw meats are packed on the same machine the equipment must be thoroughly cleaned and disinfected.

- The vacuum packing machine should be kept in good working order to ensure a proper seal is delivered.

- Packaging used on the machine should be appropriate for its use and must be stored so that it cannot be contaminated by raw meats.

- Cooked meats must be produced under hygienic conditions and should be packed as soon as possible after cooling has been completed.

- A shelf life of no greater than 10 days from the day of cooking must be given to the vacuum packed cooked meat. To ensure this can be proved you will need to introduce a date labeling system which will accompany the product through all stages.

- Extending the shelf life of such products beyond 10 days increases the risk of growth of Clostridium Botulinum and poses an unacceptable risk.

- The vacuum packed products must be stored at 5°C or below at all times during its shelf life. Temperatures above 5°C will encourage growth of Clostridium Botulinum and other bacteria.

- Vacuum packed meat needs to be protected from external contamination from raw meats and poultry and therefore should be handled with the same care as open cooked meats.

Compiled By :………………………………………..….

Approved By:…………………………………………….

Date :…………………………………………………….

GM ……………………………………………………….

Date :…………………………………………………….